SEND and RECV comprise a very simple file download utility set

for the purpose of downloading files to diskless 6C22, 4C22,

and FPWX CPU's (all V40 based).

The first requirement for SEND and RECV to work is the proper cable.

This cable has only three wires, and is properly called a data only

null modem cable. Assuming that your host machines serial port is a 9

pin male (AT pinout) type, the cable would have 9 pin female connectors

at both ends and the following connections:

9 pin      9 pin

5 -------- 5    ( ground )

2 -------- 3    ( data <- )

3 -------- 2    ( data -> )

If your host machine has a 25 pin serial connector, the cable needs a

female 25 pin connector on the host end and a female 9 pin connector on

the client end. This cable must have the following connections:

25 pin     9 pin

7 -------- 5    ( ground )

2 -------- 2    ( data -> )

3 -------- 3    ( data <- )

SEND runs on a host machine. This host machine must be a PC with a

standard COMX RS-232 serial port available.

SEND is invoked this way:

SEND PPP [BR]

PPP is the hexadecimal port address of the serial port on the host

machine (3F8 = COM1, 2F8 = COM2, 3E8 = COM3, and 2E8 = COM4)

BR is an optional baud rate parameter. if BR is not supplied, send

uses 9600 baud. For example SEND 2F8 38400 would send files through

COM2 at 38400 baud.

Once SEND is running on the host machine, RECV is run on the client

machine to download files. RECV only works on MESA V40 based CPU's!

RECV is invoked this way:

RECV RFN LFN [BR] [Q]

RFN is the remote file name (the source file) which is relative to the

path where send was launched. LFN is the local file name (the target file).

The q parameter causes SEND to be aborted when the file transfer is complete.

BR is an optional baud rate parameter. If BR is not supplied, RECV uses

9600 baud. For example RECV FOO GOO 38400 Q would get the remote file FOO,

write it to the local file GOO, and abort SEND when done. All data transfers

would be done at 38400 baud. If you set the baud rate on the command line,

the SEND baud rate must match the RECV baud rate.

Maximum baud rates: 6C22-D    : 38400

4C22,FPWX : 115200

You may not be able to use the maximum baud rates, depending on your host

CPU speed, serial port characteristics, interconnect cable etc.

(Your mileage may vary...)

####################